# An integrated model for simulation of neonatal physiology — THE mechanical support devices: ventilation and ECMO

Fifth paper in the EXPLAIN series (companion to the cardiovascular, respiratory, other-systems and AI-parameterization papers). Target journal: Pediatric Research. Markdown working draft — equations to be re-keyed as native Word (OMML) objects. Every equation is transcribed from and checked against the engine source: explain/device_models/Ventilator.js, explain/device_models/Ecls.js (and, where included, Resuscitation.js, Monitor.js). This paper builds directly on the gas-exchange and respiratory mechanics of the respiratory companion paper, which it cross-references rather than repeats.

## Abstract

(Draft — tighten to journal word limit.)

Sick newborns are frequently supported by devices that act directly on the physiology the other papers in this series describe: the mechanical ventilator, which drives gas into the lungs, and extracorporeal membrane oxygenation (ECMO), which takes over gas exchange and, in its veno-arterial form, circulatory support. We describe the device layer of EXPLAIN, a real-time whole-body neonatal physiology simulator, and its coupling to the patient. The ventilator is modelled as a gas-pressure source connected to the airway through an endotracheal tube whose resistance follows a diameter- and length-dependent law; it implements the clinically used modes — pressure control, pressure-regulated volume control, pressure support and continuous positive airway pressure — through an inspiratory/expiratory valve network with time- or flow-cycling, patient triggering and a per-breath volume-targeting servo, and it interacts with the patient’s own respiratory drive so that assisted and spontaneous breaths compose. Oxygenation and carbon-dioxide clearance are not prescribed by the device but emerge from the alveolar gas exchange of the respiratory model under the ventilation the device delivers. ECMO is modelled as a blood pump, a membrane oxygenator that reuses the same partial-pressure-driven gas-exchange equations as the native lung, and cannulae represented as resistors draining from and returning to named circulatory compartments, in veno-arterial and veno-venous configurations. We demonstrate that the ventilator restores oxygenation and ventilation in a surfactant-deficient lung as a function of inspired oxygen and airway pressure, and that ECMO supports systemic gas exchange as a function of pump flow and sweep gas. Because the devices act through the same mechanistic compartments as the native physiology, their effects — and their interactions with the patient — are emergent and interpretable.

## 1. Introduction

Much of neonatal intensive care consists of supporting, or temporarily replacing, cardiorespiratory function with machines. A mechanical ventilator delivers gas to the lungs under clinician-set pressures, volumes and timing when an infant cannot breathe adequately alone; extracorporeal membrane oxygenation (ECMO) diverts blood through an artificial lung and pump when the native lung, or the heart, cannot sustain gas exchange or the circulation at all. These are among the most consequential interventions in the newborn intensive care unit, and among the hardest to teach, precisely because their effect on the patient is never the setting itself but the interaction between the machine and a particular physiology.

That interaction is the crux. A set peak inspiratory pressure does not fix a tidal volume — it produces one that depends on the lung’s compliance and the resistance of the endotracheal tube, and that changes as the lung stiffens or recruits. A pressure-support or synchronized breath is meaningful only in relation to the infant’s own respiratory effort, which it must detect and augment rather than fight. Continuous positive airway pressure holds the lung open but ventilates only a patient who is breathing. On ECMO, pump flow and sweep-gas flow partition oxygenation and carbon-dioxide clearance between the artificial and the native lung, and in the veno-arterial configuration the returning flow interacts with the heart’s own output. In each case the clinically relevant quantity — the delivered volume, the arterial blood gas, the systemic oxygen delivery — is an emergent property of the coupled device–patient system, not a number the device sets.

A model intended to teach or investigate neonatal respiratory and circulatory support must therefore represent the devices not as generators of prescribed outputs but as physical sources — of gas pressure, or of blood flow — wired into the same compartments as the native physiology, so that the resulting mechanics, gas exchange and haemodynamics fall out of the same equations that govern the unsupported patient. That is the approach taken here. This paper describes the device layer of EXPLAIN — the mechanical ventilator, extracorporeal membrane oxygenation, and, more briefly, cardiopulmonary resuscitation and the bedside monitor — and, crucially, its coupling to the respiratory and circulatory models of the companion papers. The ventilator is a gas circuit connected to the airway through an endotracheal tube; ECMO is a blood circuit connected between two circulatory compartments whose oxygenator reuses the very same gas-exchange law as the native lung. Because the devices act through shared compartments, their effects, and their interactions with the patient’s own physiology, are emergent and interpretable. The scientific contribution is a compact, fully specified model of neonatal mechanical ventilation and ECMO and their coupling to the patient, completing the integrated simulator of the series; as throughout, device settings and patient parameters can be set by the AI-assisted calibration pipeline of the parameterization companion paper. Below we specify the devices (Section 2) and demonstrate that ventilation and ECMO restore oxygenation and carbon-dioxide clearance in a failing lung as a function of their clinical controls (Section 3).

## 2. Methods

Notation and units follow the series convention (respiratory paper §S1). Ventilator pressures are given in the clinical unit cmH₂O and converted to the engine’s mmHg by

Eq. 1   P[mmHg] = P[cmH₂O] / 1.35951

Throughout, Δt is the integration step and x ← x + (…)·Δt denotes forward-Euler integration. The gas compartments, valves (directional resistors), gas composition and the alveolar gas exchange that the devices act upon are defined in the respiratory paper and are referenced here by name.

### 2.1 The mechanical ventilator

Source: explain/device_models/Ventilator.js.

Conceptual model. The ventilator is a small gas circuit wired in series to the patient’s airway: a fresh-gas inlet compartment (VENT_GASIN) at the set inspired oxygen fraction, temperature and humidity; a compliant circuit compartment (VENT_GASCIRCUIT); an inspiratory valve (VENT_INSP_VALVE) and an expiratory valve (VENT_EXP_VALVE) (directional resistors); an expiratory reservoir (VENT_GASOUT) that sets the positive end-expiratory pressure (PEEP); and an endotracheal tube (VENT_ETTUBE) connecting the circuit to the patient’s dead space. Enabling the ventilator (switch_ventilator) opens this network and blocks the natural airway inlet (MOUTH_DS), i.e. intubates the patient; the alveolar gas exchange downstream is unchanged (respiratory paper).

Endotracheal-tube resistance. The tube is a resistor whose resistance rises with gas flow and falls with bore, following a diameter- and length-dependent law:

Eq. 2   R_ETT = max( 15, (a·f + b)·(L/L_ref) ),   a = −2.375·d + 11.9375, b = −14.375·d + 65.9374

with d the internal diameter (mm), L the length (mm, reference L_ref = 110), f the gas flow, and a floor of 15 (mmHg·s·L⁻¹). The flow-dependent term makes the tube a Rohrer-type (non-ohmic) resistance; the coefficients a, b decrease with diameter, so a wider tube offers less resistance.

Pressure control and pressure-regulated volume control (PC/PRVC). In inspiration the expiratory valve is closed and the inspiratory valve is opened with a resistance set so that the delivered flow drives the circuit toward the target peak inspiratory pressure (PIP):

Eq. 3   R_insp = (P_gasin + PIP − P_atm − PEEP) / (Q_insp/60)

(with Q_insp the set inspiratory flow, L·min⁻¹); the inspiratory valve is shut once the circuit pressure reaches PIP, so the breath is pressure-limited. In expiration the expiratory valve opens and the expiratory reservoir volume is pinned so that the circuit floats at PEEP,

Eq. 4   V_gasout = PEEP / E_base + Vᵤ

Breaths are time-cycled: the expiratory time follows from the set rate and inspiratory time,

Eq. 5   T_exp = 60/RR − T_insp

and inspiration and expiration alternate as their timers elapse. At end-expiration the dynamic compliance is measured from the achieved tidal volume and the driving pressure,

Eq. 6   C_dyn = V_T / (PIP − PEEP)  [mL·cmH₂O⁻¹]

In pressure-regulated volume control the peak pressure is adjusted breath-to-breath by a servo that drives the measured expiratory tidal volume to the set target V_T*, within bounds:

Eq. 7   PIP ← PIP ± 1 cmH₂O per breath, clamped to [PEEP + 2, PIP_max], toward V_T = V_T*

so PRVC delivers a targeted volume at the lowest sufficient pressure — the volume-targeting behaviour of the clinical mode.

Pressure support (PS). A supported breath is patient-initiated and flow-cycled: once triggered, inspiration continues while the endotracheal-tube flow rises, and the ventilator cycles to expiration when the flow falls below a fraction of its peak,

Eq. 8   cycle to expiration when f_ETT < 0.3·f_peak

reproducing the flow-termination criterion of clinical pressure support.

Continuous positive airway pressure (CPAP). Both valves remain open and the circuit is held at the CPAP level (= PEEP); the patient breathes spontaneously through the endotracheal tube (respiratory paper, the spontaneous-drive model). With spontaneous breathing off, CPAP holds airway pressure but delivers no tidal volume, as in reality. The reported minute ventilation is the patient’s own:

Eq. 9   V̇_E = V_T · RR_spont

Patient triggering (synchronized ventilation). When synchronized, a mechanical breath is triggered by the patient’s own inspiratory effort: the tube flow drawn during a spontaneous inspiration is integrated, and a breath is delivered once it exceeds a trigger volume set as a fraction of the tidal volume,

Eq. 10   trigger when ∫ f_ETT dt > V_trig = V_T · (trigger% / 100)

so that assisted breaths are phase-locked to the patient’s drive. This coupling to the spontaneous respiratory model (which reads the active airway inlet, whether the natural mouth or the endotracheal tube) is what lets the same lung be ventilated mechanically, supported, or left spontaneous, with the tidal-volume bookkeeping consistent across all three.

Reported quantities. Airway pressure is reported as (circuit pressure − atmospheric) in cmH₂O, the endotracheal-tube flow as f_ETT·60 (L·min⁻¹), and end-tidal CO₂ as the dead-space PCO₂ sampled at end-expiration. Crucially, the ventilator sets only the airway gas composition (inspired oxygen fraction) and the ventilation pattern; arterial oxygenation and carbon-dioxide clearance are not prescribed by the device but emerge from the alveolar gas exchange of the respiratory model (respiratory paper, Fick flux) under the delivered ventilation.

### 2.2 Extracorporeal membrane oxygenation (ECMO)

Source: explain/device_models/Ecls.js.

Conceptual model. ECMO is modelled as an extracorporeal blood circuit wired in series between two named patient compartments: blood is drained from a drainage site, driven by a pump through a membrane oxygenator, and returned to a return site. The model is a coordinator that owns a chain of sub-compartments — drainage cannula (a resistor) → inflow tubing → pump → oxygenator → outflow tubing → return cannula (a resistor) — together with a parallel sweep-gas limb (a gas source, an inlet valve, the oxygenator gas side, and an outlet) coupled to the blood by a membrane gas-exchanger. The veno-arterial (circulatory + respiratory support) and veno-venous (respiratory support only) configurations differ only in the choice of drainage and return sites (e.g. VA: drain the right atrium, return the aortic root; VV: drain and return on the venous side); there is no separate code path.

Pump. The pump generates a head that is a linear function of its speed, imposed as an external pressure on the node it drives (the downstream node for a centrifugal pump, the upstream node for a roller pump):

Eq. 11   p_pump = −RPM / 25  [mmHg]

(so 1500 rpm ≈ −60 mmHg). This is a deliberate surrogate rather than a physical centrifugal head–flow curve. Every circuit segment is a resistor with a prescribed resistance (the cannulae use measured resistances from a library of real devices — e.g. Bio-Medicus and Medtronic arterial/venous cannulae of stated French size — scaled by a per-segment multiplier), so the circuit flow through each segment is linear in its pressure drop:

Eq. 12   Q_i = Δp_i / R_i,   R_i = R_{0,i}·k_i

with R_{0,i} the base/library resistance and k_i the segment’s resistance factor; the pump head of Eq. 11 sets the pressure drop across the driven segment. The reported circuit flow is the return-cannula flow, 60·Q_return (L·min⁻¹).

Membrane oxygenator. Gas transfer across the membrane reuses the same partial-pressure-driven Fick flux as the native alveolar–capillary barrier (respiratory paper, Eqs. 13–14), applied between the oxygenator’s blood compartment and its sweep-gas compartment:

Eq. 13   Φ_X = (P_X,blood − P_X,gas)·D_X·Δt  (X ∈ {O₂, CO₂})

with membrane diffusion constants D_O₂ and D_CO₂; because the sweep gas is oxygen-rich and carbon-dioxide-free, the gradients drive O₂ into the blood and CO₂ out. The sweep-gas fraction of oxygen and the sweep-gas flow set the gas-side partial pressures (the inlet valve is sized to deliver the requested sweep flow), so oxygenation and decarboxylation across the membrane are governed by the same physics as the lung, differing only in the compartments and the diffusion constants.

Coupling to the patient. The circuit exchanges volume — and, with it, the dissolved gases, solutes, drugs, temperature and viscosity — with the patient at the drainage and return sites by the same advective mixing rule as the rest of the circulation (respiratory/shared Methods, Eq. S2): oxygenated blood returned to the return site and deoxygenated blood drained from the drainage site propagate their gas contents into the systemic circuit by fractional mixing. When the circuit is clamped, its sub-compartments carry no flow and the membrane exchange is disabled, so a primed but clamped circuit is inert until opened. The partition of gas exchange between the membrane and the native lung — and the haemodynamic effect of returning flow to the arterial side in the veno-arterial configuration — are therefore emergent consequences of the coupled circuit and patient, not prescribed device outputs.

### 2.3 Cardiopulmonary resuscitation and the patient monitor

Source: explain/device_models/Resuscitation.js, explain/device_models/Monitor.js.

Resuscitation. Chest compressions are modelled as a rhythmic external pressure applied to a weighted set of intrathoracic compartments (heart chambers, great vessels, lungs), following a half-rectified sinusoid:

Eq. 14   p_cc(t) = A·sin(2πf·t − π/2) + A,   A = p_max/2, f = f_comp/60

(so p_cc ramps 0 → p_max → 0 each compression). The pressure is added to each target compartment’s external pressure with a per-compartment weight, p_ext^(k) ← p_ext^(k) + w_k·p_cc; because the compartment pressures then rise, the interconnecting resistors generate forward flow (Q = Δp/R) — this is how compressions drive a circulation. Interposed ventilations are delivered through the ventilator (pressure control) with spontaneous breathing suppressed, at a set compression:ventilation pattern (switch_cpr).

Monitor. The monitor is a read-only observer that derives the displayed bedside vitals from the model state each step, writing to nothing. Heart rate is the mean of the last twelve beat-to-beat rates (60/interval); respiratory rate is the breath count over a moving window; end-tidal CO₂ is the end-expiratory dead-space PCO₂ (from the ventilator when ventilated, or the spontaneous breath peak); pre- and post-ductal saturations are read from the ascending and descending aorta and mixed-venous saturation from the venous return. Chamber and vessel pressures are reported as true integral means over the beat, p̄ = (1/n)Σp, rather than the arterial estimate (2p_min + p_max)/3, which is valid only for arterial waveforms and underestimates atrial/venous means; flows are beat-averaged, and oxygen-delivery indices are formed as flow × oxygen content. This is the block that produces the numbers the calibration pipeline (parameterization companion paper) reads as its measured targets.

### 2.4 Software implementation and AI-assisted parameterization

See shared Methods (respiratory paper §S5) for the engine implementation, and the parameterization companion paper for how device settings and patient parameters are set by the AI-assisted calibration pipeline. The devices run in the same insertion-ordered step loop as the patient models, sharing the gas and blood compartments so that support and physiology are solved together.

## 3. Results — illustrative simulations

Each experiment drives a device on a calibrated patient headlessly (shared Methods §S7) and reports the emergent response; all blood gases are produced by the engine’s gas-exchange and acid–base solvers, not prescribed by the device. Scripts: scripts/probe_ventilator.mjs (§3.1), scripts/probe_ecls.mjs (§3.2).

### 3.1 Mechanical ventilation of the surfactant-deficient lung

A preterm 28-week RDS scenario (preterm_28wk.json, ≈1 kg) was intubated and ventilated in pressure-regulated volume control with the spontaneous drive suppressed. With the tidal-volume target held at 5 mL·kg⁻¹, raising the inspired oxygen fraction raised arterial oxygenation monotonically while the tidal volume and arterial CO₂ were unchanged (Table 1a) — the device’s oxygen and ventilation controls act independently, and the volume-targeting servo held the delivered volume constant. Raising the ventilator rate raised minute ventilation and lowered arterial CO₂ across the range from hypercapnia to normocapnia (Table 1b). Raising PEEP raised the mean airway pressure and produced a smaller, monotonic improvement in oxygenation through recruitment (Table 1c). Oxygenation and CO₂ clearance are emergent from the alveolar gas exchange of the respiratory model under the ventilation the device delivers, not device outputs.

Table 1a. Oxygenation vs inspired oxygen (PRVC, target 5 mL, PEEP 5 cmH₂O, rate 40).

| FiO₂ | PaO₂ (mmHg) | SpO₂ (%) | PaCO₂ (mmHg) | V_T (mL) |
| --- | --- | --- | --- | --- |
| 0.3 | 59.4 | 92.4 | 48.9 | 5.0 |
| 0.5 | 68.1 | 94.5 | 49.4 | 5.0 |
| 0.7 | 79.7 | 96.3 | 49.9 | 5.0 |
| 0.9 | 99.7 | 97.9 | 50.3 | 5.0 |

Table 1b. CO₂ clearance vs ventilator rate (PRVC, target 5 mL, FiO₂ 0.5, PEEP 5 cmH₂O).

| Rate (min⁻¹) | Minute vol. (mL·min⁻¹) | PaCO₂ (mmHg) |
| --- | --- | --- |
| 20 | 100 | 72.5 |
| 30 | 149 | 58.7 |
| 40 | 198 | 49.4 |
| 60 | 291 | 38.3 |

Table 1c. Oxygenation vs PEEP (PRVC, target 5 mL, FiO₂ 0.4).

| PEEP (cmH₂O) | Mean airway P (cmH₂O) | PaO₂ (mmHg) | SpO₂ (%) |
| --- | --- | --- | --- |
| 2 | 2.6 | 62.9 | 93.8 |
| 4 | 4.6 | 63.5 | 93.7 |
| 6 | 6.6 | 63.9 | 93.4 |
| 8 | 8.6 | 64.4 | 93.3 |

### 3.2 ECMO support of systemic gas exchange

Severe respiratory failure was induced on a term neonate (term_neonate.json) by near-abolishing alveolar oxygen diffusion, giving profound hypoxaemia with the native lung alone (PaO₂ 11 mmHg, SpO₂ 14 %). Veno-arterial ECMO (drainage from the superior caval/right-atrial inlet, return to the aortic root) was then started. Increasing the pump speed raised the extracorporeal circuit flow and, with it, systemic oxygenation, saturating once the circuit flow plateaued at ≈ 0.47 L·min⁻¹ (≈ 130 mL·kg⁻¹·min⁻¹, a clinical neonatal flow); the post-oxygenator blood was fully saturated and mixed-venous saturation rose from 24 % to > 80 % (Table 2a). At a fixed pump speed, increasing the sweep-gas flow progressively lowered arterial CO₂ (Table 2b) — the membrane clearing carbon dioxide exactly as the sweep gradient dictates. Both effects arise from the membrane gas-exchanger (the same Fick law as the native lung) and the return of oxygenated blood to the circulation, not from prescribed device outputs.

Table 2a. ECMO rescue vs pump speed (sweep gas 0.5 L·min⁻¹, sweep FiO₂ 1.0; native lung failed).

| Pump (rpm) | Circuit flow (L·min⁻¹) | PaO₂ (mmHg) | SpO₂ (%) | SvO₂ (%) | Post-oxygenator SO₂ (%) |
| --- | --- | --- | --- | --- | --- |
| off | 0 | 11.0 | 14.2 | — | — |
| 1500 | 0.05 | 16.3 | 35.0 | 24 | 85 |
| 2500 | 0.43 | 85.3 | 98.4 | 81 | 100 |
| 3500 | 0.47 | 95.5 | 98.8 | 83 | 100 |
| 4500 | 0.47 | 98.3 | 98.9 | 83 | 100 |

Table 2b. CO₂ removal vs sweep-gas flow (pump 3500 rpm, sweep FiO₂ 1.0).

| Sweep gas (L·min⁻¹) | PaCO₂ (mmHg) | PaO₂ (mmHg) |
| --- | --- | --- |
| 0.2 | 36.7 | 97.4 |
| 0.5 | 30.5 | 94.7 |
| 1.0 | 27.0 | 90.8 |
| 2.0 | 24.6 | 87.6 |

## 4. Discussion

### 4.1 Model originality

The device layer completes the EXPLAIN series by adding the machines that neonatal intensive care uses to support the physiology the other papers describe, and it does so in a way that keeps the whole simulator internally consistent. Rather than modelling a ventilator or an ECMO circuit as a source of prescribed outputs — a set tidal volume, a set arterial oxygen tension — each device is modelled as a physical source wired into the same compartments as the native physiology: the ventilator as a gas circuit coupled to the airway through an endotracheal tube, ECMO as a blood circuit coupled between two circulatory compartments. The consequences the clinician cares about then emerge from the same equations that govern the unsupported patient. The delivered tidal volume is whatever the set pressure produces given the lung’s compliance and the tube’s resistance; the arterial blood gas on either device is computed by the same gas-exchange and acid–base solvers used everywhere else. A particularly direct expression of this economy is that the ECMO membrane oxygenator reuses the identical partial-pressure- driven flux law as the native alveolar–capillary barrier — the artificial lung and the real lung are the same equation applied to different compartments. Because the devices act through shared compartments, their interactions with the patient — a synchronized breath augmenting the infant’s own effort, veno-arterial ECMO flow interacting with native cardiac output — are represented without any special-case machinery, and remain interpretable.

### 4.2 Model validity

The simulations of Section 3 show the devices reproducing the expected clinical dependencies. Under mechanical ventilation of a surfactant-deficient lung, arterial oxygenation rose monotonically with the inspired oxygen fraction while the volume-targeting servo held the tidal volume fixed and arterial carbon dioxide constant (Table 1a), demonstrating that the device’s oxygen and ventilation controls act independently; raising the ventilator rate lowered arterial carbon dioxide across the clinical range (Table 1b); and raising PEEP produced the expected, smaller, recruitment-driven improvement in oxygenation (Table 1c). Under ECMO, a lung failed to profound hypoxaemia was rescued as pump speed raised circuit flow and systemic oxygenation, saturating at a circuit flow (≈ 130 mL·kg⁻¹·min⁻¹) in the clinical neonatal range, with the post-oxygenator blood fully saturated; and sweep-gas flow controlled carbon-dioxide removal (Tables 2a–b). In every case the blood gas is an emergent output of the coupled device–patient system rather than a device setting. As with the other companion papers, these are demonstrations of mechanistic behaviour across the operating range, not a formal clinical validation against device-specific performance data, which is future work.

### 4.3 Reproducibility and model expansion

Every quantitative result is reproduced from the engine by a named script — the ventilator sweeps by probe_ventilator.mjs and the ECMO sweeps by probe_ecls.mjs — so each table can be regenerated and audited. Each ventilator mode and each part of the ECMO circuit is a self-contained module operating on the shared gas and blood compartments, so additional support modalities can be added without modifying the engine, in the same way the ventilator and ECMO were added on top of the respiratory and circulatory models.

### 4.4 Limitations. The devices share the lumped, real-time simplifications of the rest of the

engine. The ventilator uses single-compartment lung mechanics and an empirical, flow- and bore-dependent endotracheal-tube resistance law rather than a resolved tube geometry. For ECMO, the pump is represented by a linear speed-to-pressure surrogate (Eq. 11) rather than a physical centrifugal head–flow curve, and the circuit and cannula resistances are prescribed constants (taken from measured device values) rather than computed from geometry; consequently the model captures the dependence of circuit flow on pump drive and resistance but not the detailed pressure–flow characteristic of a specific pump. The membrane oxygenator is a single lumped exchanger with fixed diffusion constants, so it does not resolve diffusion-limitation or recirculation effects. Both devices inherit the respiratory model’s lumped gas exchange (no continuous ventilation/perfusion distribution).

4.5 Future work. High-frequency oscillatory ventilation; a physical pump head–flow characteristic and geometry-derived cannula resistances; closed-loop device control (weaning, oxygen and ventilation servo-targeting) driven by the parameterization pipeline; and the interaction of veno-arterial ECMO with native cardiac recovery and differential (north–south) oxygenation.

## Conclusion

The device layer of EXPLAIN provides a transparent, real-time, mechanistic account of neonatal mechanical ventilation and extracorporeal membrane oxygenation, together with cardiopulmonary resuscitation and the derivation of the bedside monitor. By modelling each device as a physical source of gas pressure or blood flow wired into the same compartments as the native physiology — the ECMO oxygenator reusing the very gas-exchange law of the native lung — it makes the delivered volumes, blood gases and haemodynamics emergent consequences of the coupled device–patient system rather than prescribed device outputs. Together with its cardiovascular, respiratory, other-systems and parameterization companions, it completes an integrated, interpretable and freely available platform for teaching and investigating the whole of neonatal cardiorespiratory intensive care, from the unsupported patient to full mechanical and extracorporeal support.

## References

See articles/_references.md. Paper-5 sources to confirm via PubMed: an endotracheal-tube resistance reference for Eq. 2; standard references for ventilator modes (PC/PRVC/PS/CPAP) and flow-cycling; and ECMO circuit/oxygenator references. Reuse the respiratory paper’s gas-exchange citations (the ECMO oxygenator shares that law) and the series’ shared software/AI citations.