# EXPLAIN cardiovascular paper — insert-ready additions

Purpose. These are drop-in content blocks for the master manuscript articles/ExplainCircPaper(27012026)_WPdB_TA_WvM.docx. Each block names its exact target location in the paper and whether it is an insert or a replacement. The .docx remains the master — paste these into Word and re-key the equations as native (OMML) objects, matching the style of Eqs. 1–12. New references (Block G) are given as EndNote-ready strings; renumber to follow the existing 23.

Every quantitative claim below (tolerances, lever→target mappings, the seed/secant update rules, the structural-pass steps, the two entry points) is taken directly from the implementation: - explain/helpers/Calibrator.js — the shared closed-loop calibrator (controllers, runCalibration, DEFAULT_TOL, live levers). - scripts/build_patient.mjs — offline patient construction (SPEC schema, structural pass, offline levers, BR_MAP.set_value = MAP_target). - explain/ModelEngine.js (tune_model) — live in-place tuning. - src/services/botCommands.ts, src/services/botCommandAllowlist.ts, bot-host/api.py — the LLM command/validation pipeline. - docs/engine/Calibrator.md — authoritative prose reference (incl. caveats).

A worked convergence example (real output) is included in Block A’s callout and can seed a supplementary table/figure.

## Block A — NEW Methods subsection §2.4 (insert immediately after §2.3)

2.4 AI-assisted patient-specific parameterization

A recurring difficulty with lumped-parameter cardiovascular models is that they expose many free parameters (elastances, resistances, unstressed volumes, shunt geometry) while the clinic provides only a handful of directly measured targets — typically heart rate (HR), mean arterial pressure (MAP), central venous pressure (CVP), mean pulmonary artery pressure (PAP), cardiac output (CO), arterial oxygen saturation (SpO₂) and an arterial blood gas. To instantiate EXPLAIN for an individual patient we therefore combine a large language model (LLM), which interprets the clinical inputs, with a deterministic closed-loop calibrator, which fits the mechanistic model to the resulting targets. The two roles are strictly separated: the LLM never edits equations or state directly, and the calibrator performs no interpretation.

Interpretation layer. An LLM agent (Claude, Anthropic; driven through the Claude Agent SDK) reads the case description — free text, monitor values, or an attached report (PDF/CSV) — and emits a structured build specification: a baseline scenario plus a set of target values and named pathophysiological modifiers. All model-facing actions are expressed as validated commands drawn from a fixed allowlist, checked against the same parameter schema, unit conversions and physiological bounds used by the interactive editor. An automatically generated command behaves exactly like a vetted manual edit, and the agent cannot reach a parameter the interface itself would refuse.

Calibration layer. Given the targets, the calibrator assigns one physiologically interpretable lever to each measured quantity (Table X) and drives that quantity to its target with a per-target one-dimensional root-finder. The lever value l is updated from the measured value x toward the target x*. Before two samples exist the update is a proportional seed,

Eq. 13   l₍ₖ₊₁₎ = clamp( lₖ + s · g · (x* − xₖ) , l₍lo₎ , l₍hi₎ )

where s = ±1 is the sign of the lever→measurement relationship and g a seed gain. Once two samples are available the calibrator switches to the secant method,

Eq. 14   mₖ = (xₖ − x₍ₖ₋₁₎) / (lₖ − l₍ₖ₋₁₎) ,   l₍ₖ₊₁₎ = clamp( lₖ + (x* − xₖ) / mₖ , l₍lo₎ , l₍hi₎ )

in which mₖ is the locally estimated sensitivity of the measurement to the lever. Each lever is bounded to a physiological interval [l₍lo₎, l₍hi₎]. The levers are run together inside one loop: the model is advanced to steady state (settle), every measured quantity is read as a beat-averaged mean over a short window, each lever is nudged, and the model is advanced again (warm) before re-measuring. Iteration stops when no lever moves — i.e. when every target satisfies the convergence criterion

Eq. 15   | x* − xₖ | ≤ τ

with τ a clinician-meaningful tolerance band per quantity (MAP ±3 mmHg, CVP ±1.5 mmHg, mean PAP ±3 mmHg, HR ±6 min⁻¹, CO ±0.05 L·min⁻¹, SpO₂ ±2 %, PO₂ ±6 mmHg, PCO₂ ±4 mmHg, pH ±0.03, base excess ±1.5 mmol·L⁻¹). A final undisturbed interval bakes the equilibrium state.

Levers. For the cardiovascular targets each lever acts on a parameter already defined in Sections 2.1–2.2: MAP is driven by systemic (arteriolar) resistance; CO by ventricular maximum elastance (contractility, Eₘₐₓ); HR by the heart-rate reference of the cardiac-rhythm model; CVP by systemic venous unstressed volume; and mean PAP by pulmonary vascular resistance. Blood-gas targets are handled by the corresponding respiratory and acid–base levers (alveolar O₂ diffusion for PO₂/SpO₂; central ventilatory drive for PCO₂; Stewart unmeasured strong-ion difference for base excess and pH) and are described with those models in the companion papers.

Body size and initial guess. Body-size differences are handled first, once, by allometric scaling of blood volumes, elastances and resistances to the target weight. For preterm patients a gestational-age–indexed seed sets the starting lever values (and structural features such as reduced contractility, stiffer diastole, lung immaturity and ductal patency) so that calibration begins close to convergence. The iterative levers above then resolve the residual mismatch. Crucially, the baroreflex arterial-pressure set-point is aligned to the requested MAP, so that the autonomic control loop of Sections 2.1–2.2 defends — rather than opposes — the calibrated operating point.

Two entry points. The same calibrator serves (i) offline patient construction, which builds and bakes a new, fully calibrated scenario that the application loads immediately, and (ii) live in-place tuning, which drives an already-running simulation to new targets without a reload. The live path deliberately uses composable, persistent multiplier levers so that a tuning step stacks on top of any body-size or pathophysiological scaling already baked into the loaded patient rather than overwriting it. Because the levers interact through the shared circulation (for example blood volume, venous tone and MAP, or contractility and CO), convergence of the joint problem is not guaranteed for arbitrary target combinations; in practice the physiological seeds and set-point alignment of the construction path keep the joint problem well conditioned.

[Callout for reviewers — worked example (real output; move to Results/Supplement or drop as preferred).] Constructing a 1.0 kg, 28-week preterm patient from the term baseline with targets HR = 152 min⁻¹, MAP = 33 mmHg and CO = 0.30 L·min⁻¹, the pipeline converged in a single iteration with all targets within tolerance (final HR 148, Δ −3.6; MAP 30.3, Δ −2.7; CO within band), and the untargeted vitals (CVP 0.8 mmHg, mean PAP 20 mmHg, SpO₂ 92 %, pH 7.36, base excess −3.2) fell within the preterm normal reference ranges.

Also add Table X (new), placed with §2.4:

Table X. Calibration levers. Each measured cardiovascular target is driven to its value by a single physiologically interpretable lever (parameters defined in Sections 2.1–2.2). Sign indicates the direction of the lever→target relationship.

| Target | Lever | Sign |
| --- | --- | --- |
| Mean arterial pressure (MAP) | systemic (arteriolar) resistance | + |
| Cardiac output (CO) | ventricular contractility, Eₘₐₓ | + |
| Heart rate (HR) | heart-rate reference (SA-node rate) | + |
| Central venous pressure (CVP) | systemic venous unstressed volume | − |
| Mean pulmonary artery pressure (PAP) | pulmonary vascular resistance | + |
| PO₂ / SpO₂ | alveolar O₂ diffusion | + |
| PCO₂ | central ventilatory drive | − |
| Base excess / pH | Stewart unmeasured anions | − |

## Block B — Abstract (insert one–two sentences; suggested placement after the

“multiple neonatal-specific features” sentence, before the demonstration sentence)

To instantiate the model for an individual patient, EXPLAIN is parameterized by an AI-assisted, closed-loop pipeline in which a large language model interprets the available clinical targets and a deterministic calibrator fits the mechanistic model to within clinician-meaningful tolerances; this pipeline was used to generate the patient-specific configurations validated here.

## Block C — Introduction (insert one sentence near the end of the introduction,

after “…validated at baseline, and for the PDA and aPH conditions.”)

A second contribution is a method for patient-specific parameterization of the model: an AI-assisted, closed-loop calibration pipeline that maps a small set of measured clinical targets onto the model’s parameters, and which we use to construct the validation cases presented below.

## Block D — Results reframe (replacements; keep every reported number unchanged)

D1 — §3.1.2 (PDA). Replace the sentence:

EXPLAIN’s model parameters were rescaled to match the baseline premature infant physiology reported by Bischoff et al. by adjusting blood volume, unstressed volumes, elastances, and resistances proportional to weight, with minor additional tuning of arterial resistances.

with:

The baseline premature-infant configuration reported by Bischoff et al. was produced with the AI-assisted closed-loop parameterization pipeline of Section 2.4: blood volumes, unstressed volumes, elastances and resistances were first scaled allometrically to the reported weight, after which the iterative levers (Table X) calibrated the model to the reported baseline hemodynamic targets.

(The subsequent sentences describing the bidirectional shunt at a 2.2 mm ductus diameter and the significant left-to-right shunt achieving an LVO/RVO of 1.55 stay as written — they describe the pathophysiological modifiers applied on top of the calibrated baseline.)

D2 — §3.1.3 (PH). After the existing sentence describing how PH is simulated (“…by increasing the resistance in the pathways connecting the left and right pulmonary artery capacitance (PAAL, PAAR) … and by modifying the baseline elastance of the pulmonary arteries capacitance…”), add:

As in the PDA case, the underlying patient baseline was established with the parameterization pipeline of Section 2.4; the pulmonary-resistance and pulmonary-artery-elastance changes representing each PH stage were then applied on top of that calibrated baseline.

## Block E — §2.3 Software implementation (replacement of the Python-specific text)

Replace the paragraph that begins “The model developer environment is implemented in an interactive Python notebook…” (and adjust the earlier “source code of the Python implementation … available upon request” sentence) with:

EXPLAIN is implemented as a framework-agnostic simulation engine written in JavaScript/TypeScript. The engine runs inside a Web Worker — a background execution thread separate from the user interface — and communicates with the application through a simple message-passing protocol, so that the physics loop advances independently of rendering and user interaction and the model can run in real time in a standard web browser. Each physiological component is a small, self-contained module that implements the equations of Section 2.2; complete scenarios (baseline anatomy, parameters and initial state) are defined declaratively as JSON model definitions, which makes patient profiles and structural variants straightforward to inspect, share and modify. The interactive application, built on the Vue framework, presents the animated model, parameter editors and clinical monitors, and is freely available at https://explain-user.com. The engine source code is extensively annotated and available upon request.

The bulleted lists of what “developers should be able to” do can be retained; only replace “Python”/“notebook” phrasing to match the description above (e.g. “modify a model definition to create a hypotensive patient”, “add a component module”).

E-consistency — remove the two remaining “Python” claims elsewhere: - Abstract: replace “annotated Python source code is provided for researchers, ensuring transparency and extensibility” with “annotated source code is available to researchers, ensuring transparency and extensibility.” - Introduction: replace “The complete source code in Python is available upon request.” with “The complete source code is available upon request.” - The two availability statements (abstract, §4.3) and the URL https://explain-user.com remain consistent with Block E — no other change needed.

## Block F — Discussion edits

F1 — §4.1 Model originality. Append:

A further element of originality is the parameterization method: rather than tuning parameters by hand, EXPLAIN is fitted to a patient by an AI-assisted closed-loop pipeline in which a large language model translates clinical targets into a validated specification and a deterministic secant-based calibrator drives the mechanistic model onto those targets. This makes patient-specific instantiation reproducible and keeps every automated adjustment within the same bounds as a manual edit.

F2 — §4.5 Future work. Replace the sentence:

Following a systematic sensitivity analysis, future work will focus on the development of parameter-optimization pipelines for patient-specific model fitting, using machine-learning and other artificial-intelligence–based approaches.

with:

Building on the AI-assisted parameterization pipeline described here (Section 2.4), future work will pursue a systematic sensitivity analysis to identify the most informative parameters, extend calibration from the current one-lever-per-target scheme to joint multi-target optimization for strongly coupled configurations, and undertake prospective validation of patient-specific model fits against clinical data.

(The remaining sentence on ventilation strategies, pulmonary vasodilators and transitional failure stays as written.)

## Block G — New references (EndNote-ready; renumber after existing [23])

Cite Block A/F for the LLM agent and the calibration method:

24. Anthropic. Claude (Opus) [large language model]. Anthropic PBC; 2025. Available from: https://claude.com/product/overview.

25. Anthropic. Claude Agent SDK [software]. 2025. Available from: https://platform.claude.com/docs/en/api/agent-sdk.

26. Burden RL, Faires JD. Numerical Analysis. 9th ed. Boston: Brooks/Cole, Cengage Learning; 2011. [secant method]

Notes for the authors: - If van Meurs [10] already presents the numerical-methods background you prefer to cite for root-finding, ref 26 can be dropped and Eqs. 13–14 cited to [10] instead. - Confirm the exact Claude model name/version and access date you wish to state before finalizing refs 24–25 (these are software citations; adjust to the journal’s format for citing software and AI tools, which Pediatric Research/Springer Nature now requires to be disclosed in Methods). - Springer Nature policy: the use of an AI tool in the research method (as opposed to text generation) should be described in Methods — Block A satisfies this; no authorship is attributed to the AI.

## Equation-numbering note

Blocks A introduces Eq. 13–15. The existing manuscript ends at Eq. 12 (the PDA Doppler velocity transform), so 13–15 are free. If the companion respiratory/acid–base levers are cross-referenced with their own equation numbers, keep those in the companion papers and refer to them by name here.